Preparing for an Approaching Wildland Fire

The following checklists will assist you in protecting your property and the lives of your family.

Preparing your house (whether you are leaving or staying):

- Children and people with mobility problems should leave early.

- Tune into a local radio station and listen for instructions and updates.

- Place combustible patio furniture in the house or garage.

- Shut off propane at the tank, if you have one.

- Close all exterior vents.  Seal attic and ground vents with pre-cut plywood or commercial seals.

- Prop a ladder against the house so fire fighters have easy access to the roof.

- Make sure that all garden hoses are connected to faucets and attach a nozzle set on “spray.”

- Close all exterior doors, vents, windows and draw all Venetian blinds, shutters or non-combustible window coverings and heavy drapes.

- Remove lightweight and/or non-fire resistant curtains and other combustible materials from around windows.

- Attach pre-cut plywood panels to the exterior side of windows and glass doors.

- Close all interior doors.

- Open the fire place damper, but place the screen over the hearth to prevent sparks and embers from entering the house.

- Leave a light on in each room to increase the visibility of your home in heavy smoke.

- Turn off all pilot lights.

- Move overstuffed furniture (such as couches, easy chairs, etc.) to the center of the room.

Preparing yourself and others to stay

- Evacuate, if possible, all family members not essential to protecting the house, as well as all pets and valuables.

- Assign tasks to all family members who will be protecting the house.  These tasks should all be assigned and practiced, if possible, prior to an event.

- Contact a friend or relative outside the area and relay your plans.

- Arrange a meeting place well away from the fire and make sure family members are aware of its location

- Place vehicles in the garage, pointing out, and roll up the windows.

- Place valuable papers and mementoes in the car.

- Close the garage door, but leave it unlocked.  If you have an electric garage door, disconnect it so that the door can be opened manually.

- Wear only cotton or wool clothes.  Proper attire includes long pants, long sleeved shirt or jacket and boots.  Carry gloves, a handkerchief to cover your face, water to drink, and goggles.

- Soak rags, towels or small rugs with water to use in beating out embers or small fires.

- Inside, fill bathtubs, sinks and other containers with water.  Outside, do the same with garbage cans and buckets.  Remember that the water heater and toilet tank are available sources of water.

- Gather fire tools, such as rakes, shovels, hoses, ladders and hoes.

- Continually check the roof and attic for embers, smoke, or fire.

- As the flame front approaches go inside, wait for it to pass then come out to extinguish fire.

# What you can reasonably expect if you decide to stay

History tells us that a high percentage of people killed in wildfires are killed in vehicle-related events when they try to leave the area too late and find that:

Visibility is restricted resulting in a disabling crash

Finding escape roads are blocked by other vehicles, trees or debris

You become disabled in an area without a safety zone and get burned over

If you stay at your home or place of safety you may survive and minimize the damage to your property.  BUT:

It depends on what the fire conditions are, what the fuel load is, what preparations you have already made and some luck.

Consider the following if you are thinking of staying.

- Rescue by first responders is unlikely.

- The wind will be extreme—think hurricane force

- Power will go out.

- Emergency generators may fail from plugged filters.

- Your well pump will not have power to provide water.

- Energized power lines will be on the ground.

- Visibility will go from bad to black.

- Flying embers will inundate the area.

- Trees will fall blocking roads and escape routes.

- It will get incredibly hot and difficult to breathe. (fire front is 1600 to 2000 degrees)

180-200 degrees will destroy your lung tissue

- It will be very loud and communication will be difficult.

- Phone and cellular lines will be busy or not available.

- Propane tanks will be venting fire and gas and can explode.

- Other people will do unpredictable things.

- The fire will do unpredictable things.

If you are not completely ready, then leave early—It is rude to make the deputies worry about you